# Model Evaluation & Comparative Analysis: Network Congestion Prediction

Overview: This report compares five computational models that try to predict network congestion. Each model was trained on a UDP-burst congestion scenario (sudden bursts of traffic) and then tested out-of-distribution on an unseen gradual-ramp scenario (traffic that slowly increases). Testing on a scenario the models have never seen shows which ones truly understand congestion and which ones only memorized the training data. The comparison explains why each model is used or avoided.

In simple terms: Each model is judged on two tasks: (1) an early-warning alarm, a yes/no prediction of whether congestion is coming, and (2) a latency forecast, a prediction of the actual delay value. No single model wins both tasks, so we recommend a hybrid model that combines the strengths of two of them

The terms below are used throughout the report.

| UE (User Equipment) | A device, such as a phone, that connects to the network. |
| --- | --- |
| Cell | The area covered by one base station. Devices in the same cell share its radio resources, so they can compete with each other. |
| Latency | The delay before data arrives. Higher latency means a slower, more congested network. |
| Alarm (binary early warning) | A yes/no prediction of whether congestion is about to happen. |
| Latency forecast | A prediction of the actual latency number for the near future. |
| HARQ retransmission | How often the radio link has to resend data that arrived with errors. It rises when the network is stressed. |
| ROC-AUC (alarm quality) | How well the model separates congested moments from normal ones. 0.5 is random guessing; 1.0 is perfect. |
| F1 score (alarm quality) | A balance between catching real congestion and avoiding false alarms. 0 is worst; 1.0 is perfect. |
| R² (forecast quality) | How close the predicted numbers are to the real ones. 1.0 is perfect; 0 means no better than always guessing the average; a negative value means worse than guessing the average. |
| Out-of-distribution test | Testing on a scenario that was not in the training data, to check whether a model generalizes. |

# Performance Comparison

| Model Family | Model | ROC-AUC (Alarm) | F1 Score (Alarm) | Latency R² (Forecast) | Recommended Primary Role |
| --- | --- | --- | --- | --- | --- |
| Graph Neural | GraphSAGE GNN | 0.924 | 0.535 | 0.039 | Binary Early-Warning Alarms |
| Convolutional | TCN | 0.880 | 0.507 | 0.775 | Balanced / Fast Parallel Training |
| Recurrent | LSTM | 0.879 | 0.482 | 0.832 | Numeric Forecasts & SLA Dashboards |
| Statistical | Moving Average | 0.890 | 0.505 | 0.269 | Statistical Floor Baseline |
| Statistical | ARIMA (2,0,1) | 0.798 | 0.392 | -261.96 | Unstable / Do Not Use |

KEY TAKEAWAY

The GNN is the best at raising alarms. LSTM and TCN are the best at predicting the actual latency numbers. The simple Moving Average is a surprisingly tough baseline. We therefore recommend a Hybrid Model:

A GNN spatial encoder (which looks at neighboring devices) combined with an LSTM/TCN temporal sequence layer (which looks at each device's history), so the model gets both strengths.

# Final Strategic Conclusion: The Hybrid Model

To remove the trade-off between alarm accuracy and numeric reliability, the project proposals recommend developing a Hybrid Spatio-Temporal Model. It has two main design choices:

Architecture: Combine a GNN spatial aggregation layer (captures how neighboring devices in a cell compete for resources) with an LSTM or TCN temporal encoder (tracks the history of each single device).

Loss Objective: Train the whole network end-to-end directly on a binary classification loss such as Binary Cross-Entropy (a standard error measure for yes/no predictions). This is better than predicting a score first and then applying a cutoff (post-hoc sigmoid thresholding) afterwards.

# References

- Bai, S., Kolter, J. Z., & Koltun, V. (2018). An empirical evaluation of generic convolutional and recurrent networks for sequence modeling. arXiv. https://arxiv.org/abs/1803.01271

- Box, G. E. P., Jenkins, G. M., Reinsel, G. C., & Ljung, G. M. (2015). Time series analysis: Forecasting and control (5th ed.). Wiley.

- Guo, S., Lin, Y., Feng, N., Song, C., & Wan, H. (2019). Attention based spatial-temporal graph convolutional networks for traffic flow forecasting. Proceedings of the AAAI Conference on Artificial Intelligence, 33(01), 922–929.

- Hamilton, W. L., Ying, R., & Leskovec, J. (2017). Inductive representation learning on large graphs. Advances in Neural Information Processing Systems, 30, 1024–1034.

- Hochreiter, S., & Schmidhuber, J. (1997). Long short-term memory. Neural Computation, 9(8), 1735–1780.

- Li, Y., Yu, R., Shahabi, C., & Liu, Y. (2018). Diffusion convolutional recurrent neural network: Data-driven traffic forecasting. International Conference on Learning Representations (ICLR 2018).

- Yu, B., Yin, H., & Zhu, Z. (2018). Spatio-temporal graph convolutional networks: A deep learning framework for traffic forecasting. Proceedings of the 27th International Joint Conference on Artificial Intelligence (IJCAI-18), 3634–3640.